Korean Thermal Coal Market

Korea Market Fundamentals

A compact primer on Korea's power-market structure, coal/LNG switching drivers, and why KPX dispatch matters for thermal coal.

| Field | Value |
| --- | --- |
| Audience | Thermal coal trader or analyst |
| Purpose | Understand Korea power-market fundamentals before reading scenario outputs |
| Scope | Institutions, KPX cost-based pool, 2025 fuel data, policy overlays, and trader implications |
| Date | 2026-06-25 |

# Executive Read

Core market point Korea is not a simple merchant coal-versus-JKM switch. KPX runs a centrally scheduled Cost-Based Pool; KEPCO is the single buyer; generator costs are assessed; and coal burn is shaped by nuclear output, LNG availability, fine-dust restrictions, carbon recognition, and system constraints as much as by spot coal price.

The commercial coal question is not just whether coal is cheaper than LNG. The better question is whether coal can physically and legally run, and whether its recognized cost position is changing versus LNG.

Coal produced 164.8 TWh of KPX-traded electricity in 2025, close to LNG at 159.2 TWh. That makes small monthly shifts meaningful for prompt cargo demand. [S1]

LNG set the mainland SMP in most counted 2025 intervals, but coal still carried large baseload energy. Price setting and fuel burn are different signals. [S1]

Winter fine-dust policy can restrict coal even when coal is in merit; nuclear outages can lift fossil demand even when coal/LNG spreads do not move. [S10], [S14]

# 1. Market Architecture

KPX operates Korea's wholesale power market and system operation. KPX describes the market as a pool in which generators sell electricity and KEPCO acts as the single buyer. The IEA describes the structure as a day-ahead wholesale market run by KPX, with KEPCO controlling transmission, distribution, and retail supply. [S2], [S7]

| Actor | What they control | Why a coal trader should care |
| --- | --- | --- |
| KPX | Market operation, system scheduling, SMP calculation, settlement administration | Determines day-ahead schedules and marginal pricing under cost-based rules. |
| KEPCO | Single buyer, transmission/distribution/retail monopoly role | Retail pricing and wholesale cost recovery do not work like a fully liberalized merchant market. |
| KEPCO gencos | Large coal and LNG fleet ownership | Their outages, fuel costs, and policy compliance affect aggregate coal burn. |
| KHNP | Nuclear fleet | Nuclear output is a major residual-load driver for fossil generation. |
| KOGAS/direct LNG importers | Gas procurement and supply channel | LNG economics may reflect portfolio procurement or direct-import economics, not spot JKM alone. |
| MOTIE/MCEE | Long-term supply plan, energy policy, fine-dust policy | Can shift coal availability independently of price spreads. |

Source: KPX and IEA market structure descriptions. [S2], [S7]

# 2. KPX Cost-Based Pool

Korea's wholesale market is a Cost-Based Pool. KPX states that generator fixed costs and variable costs are examined monthly by the Generation Cost Assessment Committee based on documents submitted by generators. The Cost Evaluation Committee performs the assessed-cost function. [S2], [S5]

KPX determines market prices one day ahead for each hour. Its Price Setting Schedule selects generator operation to meet forecast demand while satisfying system constraints at the lowest generation cost; the highest-cost dispatched generator becomes the marginal plant and sets SMP. [S3]

The trading-system description matters because KPX explicitly refers to schedules that consider fuel and transmission constraints. A pure coal/Japan-Korea Marker comparison misses that operational layer. [S4]

| Term | Meaning | Trader relevance |
| --- | --- | --- |
| GCAC | Monthly assessment of generator costs | Fuel-price changes can lag in recognized dispatch economics. |
| PSS | Price Setting Schedule | The relevant stack is assessed-cost dispatch, not free bids. |
| SMP | System Marginal Price | Frequently set by LNG, but not equal to LNG generation share. |
| Settlement | Post-trade cash process | Settlement has more components than energy price; burn and payments are not the same thing. |

Source: KPX trading, price-determination, and settlement process pages. [S2]-[S6]

# 3. 2025 Fuel Baseline

| Fuel | Capacity MW | Trading GWh | Share of trading volume |
| --- | --- | --- | --- |
| Nuclear | 26,050 | 175,549 | 32.1% |
| Coal | 41,739 | 164,790 | 30.1% |
| LNG | 48,388 | 159,227 | 29.1% |
| Renewables | 19,457 | 39,697 | 7.3% |
| Pumped storage | 4,700 | 4,414 | 0.8% |
| Oil / other | 5,824 | 3,485 | 0.6% |

Source: KPX 2025 Power Market Statistics. [S1]

The key trading read is that coal and LNG were close in annual traded energy. Coal was not marginal most of the time, but it was still a large physical baseload consumer. That distinction is where a coal trader can have edge: SMP tells you marginal price setting; monthly fuel-burn data tells you physical demand.

| Month | Coal GWh | LNG GWh | What it suggests |
| --- | --- | --- | --- |
| January | 12,709 | 15,345 | Winter coal restrictions and LNG/heating season matter. |
| April | 7,936 | 13,198 | Shoulder-month coal burn was low versus summer. |
| July | 19,582 | 13,720 | Summer demand/nuclear profile supported coal burn. |
| August | 20,208 | 14,853 | Highest 2025 monthly coal burn in the KPX baseline. |
| December | 16,660 | 13,261 | Winter demand supported coal, but seasonal policy can still cap upside. |

Source: KPX monthly electric energy trading volume by fuel type. [S1]

# 4. What Moves Korean Coal Burn

| Driver | Bullish for coal burn when... | Bearish / switch risk when... |
| --- | --- | --- |
| Power demand | Weather or industrial load lifts monthly fossil need. | Demand softens in shoulder months. |
| Nuclear output | Outages reduce low-cost baseload and raise residual fossil demand. | High nuclear availability compresses coal and LNG burn. |
| Renewables | Weak solar/wind increases residual demand. | Strong renewables reduce fossil need, especially around solar-heavy hours. |
| Coal restrictions | Caps relax or seasonal controls are not binding. | Fine-dust season constrains coal units regardless of fuel spread. |
| LNG price | LNG rises relative to coal, widening coal's merit advantage. | LNG falls enough to displace the switchable coal band. |
| FX | KRW strength lowers imported-fuel cost; relative effect depends on commodity move. | KRW weakness raises both imported coal and LNG costs. |
| Carbon recognition | Carbon is low or not fully reflected in dispatch economics. | Carbon is recognized strongly; coal is more CO2-intensive than LNG. |

# 5. Fuel Economics Translation

Coal traders think in USD/t and calorific value, while KPX-style dispatch economics require KRW/kWh. The bridge is energy content, heat rate, FX, variable O&M, and carbon.

coal USD/MMBtu = coal USD/t / MMBtu per tonne

fuel KRW/kWh = USD/MMBtu * USD/KRW * heat rate MMBtu/MWh / 1000

carbon KRW/kWh = tCO2e/MWh * KRW/tCO2e * carbon recognition % / 100 / 1000

MOTIE's 11th Basic Plan materials provide Korea-specific average power-sector emissions factors used for coal and LNG comparison: coal 0.8384 tCO2e/MWh and LNG 0.3800 tCO2e/MWh. [S8]

# 6. Policy and Structural Overlays

Fine-dust seasonal management is important because it can make coal availability a policy variable. The English ministry release describes stronger measures from December 1 to March 31, including coal-unit shutdowns and output caps in the referenced plan. [S10]

K-ETS is also relevant, but carbon cost does not always translate one-for-one into dispatch in a regulated cost-based market. ICAP describes K-ETS coverage and Phase 4 rules; the IEA highlights that Korea's cost-based market historically limited direct carbon-price reflection in dispatch and retail signals. [S11], [S12]

Long-term supply planning matters for the trade because more nuclear and renewables structurally reduce fossil residual demand, while coal retirements/conversions reduce coal's upper bound. The 11th Basic Plan covers 2024-2038 and includes new nuclear capacity, renewable expansion, and a shifting generation mix. [S8], [S9]

# 7. Questions a Trader Will Ask

Is this burn or marginal price? Answer: treat KPX monthly fuel generation as physical burn proxy; SMP tells marginal price-setting, not coal consumption.

Does this use customs imports? Answer: no. It uses KPX-traded generation; imports/inventory/procurement timing are separate layers.

Where can edge come from? Answer: nuclear outage expectations, fine-dust/cap assumptions, LNG relative value, and month-specific fossil residual demand.

Why not just use spark/dark spread? Answer: Korea's cost-based dispatch, recognized-cost timing, coal floors/caps, and policy restrictions make a simple spread incomplete.

# References

[S1] Korea Power Exchange (KPX), 2025 Power Market Statistics, published 2026-03-31. https://www.kpx.or.kr/boardDownload.es?bid=0045&list_no=77084&seq=4

[S2] KPX, Electricity Market Trading Process. https://www.kpx.or.kr/menu.es?mid=a20201000000

[S3] KPX, Market Price Determination. https://www.kpx.or.kr/menu.es?mid=a20203000000

[S4] KPX, Electricity Market Trading System. https://www.kpx.or.kr/menu.es?mid=a20202000000

[S5] KPX, Electricity Market Operation Council / Cost Evaluation Committee. https://new.kpx.or.kr/menu.es?mid=a20205000000

[S6] KPX, Settlement Process. https://new.kpx.or.kr/menu.es?mid=a20204000000

[S7] IEA, Korea Electricity Security Review. https://www.iea.org/reports/korea-electricity-security-review

[S8] MOTIE, 11th Basic Plan for Long-term Electricity Supply and Demand announcement. https://www.motir.go.kr/kor/article/ATCLc01b2801b/70083/view

[S9] KDI Economic Information and Education Center, MOTIE 11th Basic Plan confirmation release. https://eiec.kdi.re.kr/policy/materialView.do?num=263534

[S10] Ministry of Climate, Energy and Environment, Fine Dust Seasonal Management release. https://mcee.go.kr/eng/web/board/read.do?boardId=1640870&boardMasterId=522&menuId=461

[S11] ICAP, Korea Emissions Trading System factsheet. https://icapcarbonaction.com/en/ets/korea-emissions-trading-system-k-ets

[S12] IEA, ETS in the power sector, Korea case discussion. https://www.iea.org/reports/implementing-effective-emissions-trading-systems/ets-in-power-sector

[S13] IEA, Natural Gas Supply Security in Korea. https://iea.blob.core.windows.net/assets/4240647e-42f1-4791-8b12-59a09ca8fff9/NaturalGasSupplySecurityinKorea.pdf

[S14] IEA, Korea 2025 Energy Policy Review, executive summary. https://www.iea.org/reports/korea-2025/executive-summary