Korean Thermal Coal Market

Model Walkthrough

How Monthly Burn works, with archived intraday stack notes, trader-focused definitions, and interpretation guidance.

| Field | Value |
| --- | --- |
| Audience | Thermal coal trader or analyst |
| Tools | Monthly Burn and archived KPX Cost Stack Lab |
| App URL | https://qrlow.github.io/kpx-cost-stack-lab/ |
| Date | 2026-06-25 |

# One-Minute Pitch

What the project is really showing Monthly Burn converts Korean power-market fundamentals into a coal trader's language: tonnes, cargoes, 3-month demand, LNG switch risk, and coal price headroom. It is not trying to be a black-box forecast. It is a transparent scenario engine that shows which market lever would change Korean thermal coal demand.

Default settings reproduce the selected 2025 KPX monthly coal/LNG generation baseline instead of forcing all coal to run just because it is cheaper on paper.

Scenario changes show incremental coal burn versus that baseline, which is the part a trader can map to tender timing, cargo demand, and relative-value risk.

The model separates physical constraints from price incentives: coal can be cheap and still capped; LNG can be expensive and still needed for residual load.

# 1. Monthly Model: What It Answers

The monthly model asks: for a selected delivery month, how much coal-fired generation does Korea need versus its 2025 KPX baseline, and what does that imply for thermal coal tonnes and cargo equivalents?

| Output | What it means | Trader use |
| --- | --- | --- |
| Coal burn | Modeled coal-fired generation in GWh and converted million tonnes | Physical demand read, not just price signal. |
| Cargo equivalent | Coal tonnes divided by selected cargo size | Turns burn delta into prompt cargo/tender scale. |
| 3-month strip | Selected month plus next two months | Matches procurement windows better than a single-month view. |
| Coal headroom | Break-even coal price versus LNG minus current coal price | Measures how much coal can rally before LNG starts threatening the switchable band. |
| Fuel cost spread | LNG variable cost minus coal variable cost | Positive means coal is cheaper; negative means LNG is cheaper. |
| Trader signal | Balanced, tender support, strong coal bid, soft demand, or LNG switch risk | Quick screening label for the scenario. |

# 2. Monthly Engine Step-by-Step

Start from actual 2025 KPX monthly traded generation by fuel: nuclear, coal, LNG, renewables, pumped storage, oil, other, and total load proxy. [S1]

Apply scenario shocks to total demand, nuclear output, and renewable output. These determine monthly fossil residual need.

Keep oil, pumped storage, and other generation as fixed baseline blocks for simplicity.

Calculate fossil need: total power demand minus nuclear, renewables, and fixed other generation.

Calculate coal maximum energy from coal capacity, month hours, coal availability, and any Dec-Mar seasonal restriction stress.

Calculate coal floor energy from baseline coal burn times the Coal Floor percentage. This is the physical-inflexibility anchor.

Allocate incremental fossil need to coal or LNG based on variable cost order, subject to coal max, coal floor, and LNG availability.

Apply economic switching only to the switchable fossil band, and only when coal's relative advantage changes versus the default reference case.

Convert coal GWh to tonnes and cargoes, then aggregate the selected month and next two months for the strip view.

# 3. Physical Inflexibility of Coal Units

Coal plants cannot behave like perfectly flexible financial spread options. They have minimum stable generation, slower start/stop dynamics, planned maintenance, fuel scheduling, environmental limits, and operating commitments. The monthly model captures this with two constraints.

| Constraint | Model representation | Interpretation |
| --- | --- | --- |
| Coal maximum | coal capacity MW * 24 * days * coal availability %; optionally derated in Dec-Mar when seasonal restriction stress is on | Coal cannot exceed available fleet energy even if it is very cheap. |
| Coal floor | baseline monthly coal GWh * Coal Floor % | A portion of coal burn is treated as sticky and not easily displaced by LNG. |
| Switchable fossil band | (baseline coal + baseline LNG) * Switchable Fossil Band % | Only this band can move on relative fuel economics in the monthly screen. |
| Switch intensity | absolute shift in coal advantage / 12 KRW/kWh, capped at 100% | Small price changes move part of the band; large price changes move all contestable energy. |

How to explain this to a trader The model does not say every coal MWh switches the moment LNG gets cheaper. It says there is a sticky coal base, a physical coal ceiling, and a contestable fossil band. That is closer to how monthly burn and procurement actually behave.

# 4. Fuel Cost Spread

Fuel cost spread is the model's simple coal-versus-LNG merit read.

fuel cost spread = LNG variable cost KRW/kWh - coal variable cost KRW/kWh

Positive spread: LNG is more expensive than coal, so coal has dispatch-cost advantage.

Negative spread: LNG is cheaper than coal, so LNG switch risk rises.

It includes fuel, heat rate, variable O&M, FX, carbon price, and carbon recognition.

It is not the same thing as observed SMP. SMP is a market price; this spread is a relative variable-cost screen.

# 5. Coal Headroom

Coal headroom is the most trader-friendly price output. It answers: how far can the coal price rise before modeled coal loses its variable-cost advantage to LNG?

coal headroom USD/t = break-even coal USD/t versus LNG - current coal USD/t

Positive headroom means coal is still cheaper than LNG by that many USD/t under the scenario.

Near-zero headroom means the trade is close to the LNG switch boundary.

Negative headroom means coal is out of merit versus LNG before considering physical floors, caps, or policy constraints.

Headroom changes with LNG price, FX, coal NAR, heat rates, carbon price, and carbon recognition.

The break-even calculation backs out the coal fuel price that would make coal variable cost equal LNG variable cost after coal O&M and recognized carbon are included.

# 6. Cargo Equivalent

Cargo equivalent converts power burn into a unit coal traders actually think about.

coal Mt = coal GWh * 1000 MWh/GWh * coal heat rate / MMBtu per tonne / 1,000,000

cargo equivalents = coal Mt * 1000 / cargo size kt

If the model shows +0.45 Mt versus baseline and the cargo size is 150 kt, that is roughly +3 cargoes. This is a scale indicator, not a named tender forecast. It helps decide whether a scenario is commercially material.

# 7. 3-Month Strip

The 3-month strip is the selected month plus the next two months. If August is selected, the strip is August/September/October. This approximates how traders think about procurement and delivery windows better than a single calendar month.

Use the monthly number for immediate burn sensitivity.

Use the 3-month strip for cargo program pressure and whether the story is persistent.

A one-month spike may be weather/noise; a strip increase is more relevant to procurement and relative value.

# 8. Monthly Controls

| Control | What it changes | Question it answers |
| --- | --- | --- |
| Delivery Month | Selects KPX 2025 monthly baseline and days in month | Is this a summer, winter, or shoulder-month setup? |
| Power Demand vs 2025 | Scales total monthly demand | Is weather/industrial load adding fossil burn? |
| Nuclear Output vs 2025 | Scales nuclear generation | Are outages lifting coal/LNG residual demand? |
| Renewables vs 2025 | Scales renewable generation | Is weak renewable output adding fossil demand? |
| Coal Availability | Caps maximum coal energy | Can coal physically run more? |
| Coal Floor | Defines sticky coal burn | How much coal is hard to displace? |
| Switchable Fossil Band | Defines coal/LNG volume exposed to price switching | How much burn is actually contestable? |
| Coal/LNG/FX/Carbon | Sets variable-cost comparison | Is coal still in merit versus LNG? |
| Cargo Size | Converts Mt into cargo count | What is the tender/cargo scale of the change? |
| Seasonal Restriction Stress | Applies Dec-Mar coal derate | Does fine-dust season cap coal upside? |

# 9. Intraday Stack in One Page

The intraday stack is a one-hour dispatch teaching tool. It shows how recognized fuel costs, nuclear availability, renewables, coal caps, reserve scarcity, and fine-dust constraints can determine the marginal fuel and modeled SMP for an hour.

It ranks representative generation blocks by variable cost after mandatory and policy constraints.

It includes coal minimum-stable output as a must-run thermal block.

It shows why LNG often sets price even when coal produces a large amount of energy.

It is useful for explaining KPX mechanics, while the monthly model is more useful for cargo-demand discussion.

# 10. Model Interpretation Notes

| Topic | Interpretation |
| --- | --- |
| Is this a forecast? | No. It is a transparent scenario model anchored to KPX 2025 monthly data. Its value is decomposition: what lever changes Korean coal burn and by how much. |
| Why not customs import data? | Imports reflect procurement timing, inventory, quality, and stock changes. KPX burn is closer to power-sector consumption; imports are the next layer to add. |
| Why does default match 2025 instead of maxing cheap coal? | Because the baseline already embeds real constraints and operations. Only changes versus the reference case move the contestable band. |
| What does negative headroom mean? | At the chosen LNG, FX, carbon, and heat-rate assumptions, coal is above the LNG break-even and exposed to LNG displacement, subject to coal floor and constraints. |
| What is the biggest weakness? | No unit-level outage stack, no inventory/tender data, and no live price feeds yet. Those are obvious upgrades rather than hidden assumptions. |
| Why is this useful? | It turns a Korea power-market story into trader units: Mt, cargoes, strip demand, and switch risk. |

# 11. Current Model Assumptions

| Assumption | Current value | Rationale |
| --- | --- | --- |
| Coal heat rate | 9.15 MMBtu/MWh | Representative monthly fleet heat rate used for burn conversion. |
| LNG heat rate | 7.10 MMBtu/MWh | Representative combined-cycle monthly comparison point. |
| Coal emissions factor | 0.8384 tCO2e/MWh | MOTIE Korea-specific average factor. [S8] |
| LNG emissions factor | 0.3800 tCO2e/MWh | MOTIE Korea-specific average factor. [S8] |
| Default carbon price | KRW 16,700/tCO2e | Illustrative OPIS/MarketWatch KAU25 reference. [S15] |
| Contestable switch threshold | 12 KRW/kWh for full switch band | Screening parameter; not a KPX rule. |
| Procurement window | 3 months | Trader-oriented strip view. |

# 12. Limitations and Upgrade Path

Not a confidential KPX production-cost model.

No unit-level outage schedule, ramp-rate optimization, minimum up/down time, or network constraints.

No customs imports, utility inventories, or tender-by-tender procurement layer.

No automated live coal/LNG/FX/carbon data feed yet.

Best next upgrade: add import/inventory/tender data and a live commodity-price snapshot, then compare modeled burn pressure to actual procurement behavior.

# References

[S1] Korea Power Exchange (KPX), 2025 Power Market Statistics, published 2026-03-31. https://www.kpx.or.kr/boardDownload.es?bid=0045&list_no=77084&seq=4

[S2] KPX, Electricity Market Trading Process. https://www.kpx.or.kr/menu.es?mid=a20201000000

[S3] KPX, Market Price Determination. https://www.kpx.or.kr/menu.es?mid=a20203000000

[S4] KPX, Electricity Market Trading System. https://www.kpx.or.kr/menu.es?mid=a20202000000

[S8] MOTIE, 11th Basic Plan for Long-term Electricity Supply and Demand announcement. https://www.motir.go.kr/kor/article/ATCLc01b2801b/70083/view

[S10] Ministry of Climate, Energy and Environment, Fine Dust Seasonal Management release. https://mcee.go.kr/eng/web/board/read.do?boardId=1640870&boardMasterId=522&menuId=461

[S11] ICAP, Korea Emissions Trading System factsheet. https://icapcarbonaction.com/en/ets/korea-emissions-trading-system-k-ets

[S12] IEA, ETS in the power sector, Korea case discussion. https://www.iea.org/reports/implementing-effective-emissions-trading-systems/ets-in-power-sector

[S15] MarketWatch / Dow Jones OPIS, South Korea Approves Market Stability Reserve for K-ETS. https://www.marketwatch.com/story/south-korea-approves-market-stability-reserve-for-k-ets-opis-31412180

[S16] GitHub repository for KPX Cost Stack Lab. https://github.com/qrlow/kpx-cost-stack-lab